Electronics and Electrical Communications Engineering
Department

Faculty of Engineering

Cairo University

Submitted to: Dr. Mahmoud El-Hadidi

AquaVision: Scalable IoT-Based Smart Aquaculture Management System with Individual Fish Tracking

Initial Report submitted for course ELC4015 “Selected Topics in Communications: Internet of Things“

4th Year

1st Semester - Academic Year 2025/2026

Prepared by:

Team 3

| NAME | SECTION | ID |
| --- | --- | --- |
| Yousef Khaled Omar Mahmoud | 4 | 9220984 |
| Mohammed Hatem Ahmed Mahmoud | 3 | 9220676 |
| Ahmed Mohamed Ahmed Soltan | 1 | 9220080 |

Submission Date: 29 November 2025

# Table of Contents

1-	Table of Contents	2

2-	Table of Figures	3

3-	List of tables	3

4-	Executive Summary	4

5-	Name of IoT Vertical Application	4

6-	Functions Performed by the IOT System	4

6.1	Individual Fish Tracking via LF-RFID	4

6.2	Multi-Tank Water Quality Monitoring	5

6.3	Hierarchical Data Aggregation (Star Topology)	6

6.4	AI-Powered Water Quality Optimization	7

6.5	Predictive Disease Detection and Prevention	7

6.6	Automated Feeding Optimization	8

6.7	Dual User Interface System	9

6.8	Scalable Commercial Deployment	10

7-	TYPE OF DATA COLLECTED	11

7.1	Per-Tank Sensor Data	11

7.2	Fish Tracking Data (LF-RFID)	12

7.3	Collector Aggregated Data	13

7.4	Central Node to Cloud Data	14

7.5	Data Format – JSON Sensor Snapshot	14

7.6	Transmission Protocol	15

7.7	Bandwidth & Storage Evaluation	16

8-	Information Presentation to End-User	16

8.1	Central Node Touchscreen Dashboard (On-Site Interface)	16

8.2	Mobile Application Interface (Owner Remote Access)	19

8.3	AI Conversational Assistant	19

8.4	Alert Notification & Escalation System	20

9-	Detailed Schematic Diagram for the Overall System	20

9.1	Physical Deployment Layout	20

9.2	Sensor Placement and Radio Coverage	21

9.3	Central Node & Backhaul Connectivity	21

9.4	System Architecture	22

9.5	Type and Location of Sensors	24

9.6	Power Budget and Energy Model	25

9.7	Type & Location of Actuators	25

9.8	Type & Location of Gateways — Network Layer Distribution	26

9.9	Communication Technique — Sensor Nodes → Gateway	27

9.10	Communication Technique — Gateways → Central Node	27

9.11	Communication Technique — Internet Link (Central → Cloud)	27

9.12	Software Protocol for Data Transfer (Cloud + Edge)	28

9.13	Backend Servers for Storage & Processing	28

9.14	Human Interface Layer (Man–Machine Interaction)	28

10-	References:	29

10.1	Appendix A: Acronyms and Abbreviations	29

# Table of Figures

Figure 1 Dashboard Overview	18

Figure 2 Mobile Dashboard Overview	19

Figure 3 AQUAVISION FARM PHYSICAL LAYOUT (TOP VIEW)	20

Figure 4 AquaVision Four-Tier Architecture	23

Figure 5 Sensors Connections	24

Figure 6 Actuator Layout on Physical Farm	25

Figure 7 Gateway Placement	26

List of tables

Table 1 Per-Tank Environmental Sensor Data Specifications	11

Table 2 Fish Tracking RFID Data Specifications	12

Table 3 Collector Node Aggregated Data Structure	13

Table 4 Central Node to Cloud Data Transmission Structure	14

Table 5 MQTT Advantages for IoT Communication	15

Table 6 Daily Data Volume	15

Table 7 Cloud Download	15

Table 8 System-Wide Data Volume Summary	16

Table 9 Cloud Cost	 [2] [3]	16

Table 10 Bandwidth Optimization	16

Table 11 Hardware Configuration	17

Table 12 Software Architecture	17

Table 13 AI Capabilities	19

Table 14 Alert Channels	20

Table 15 Automated Intervention Examples	20

Table 16 Zone Description	21

Table 17 Zone Coverage	21

Table 18 Sensor Location	24

Table 19 Baseline Power Characteristics [4] [5]	25

Table 20 Acuator Location	26

Table 21 Gateway unction	27

Table 22 LoRaWAN Advantages [5] [6]	27

Table 23 4G LTE Benefits [7]	28

Table 24 MQTT Features [8]	28

Table 25 Backend Services [9]	28

Table 26 Acronyms and Abbreviations[1]	29

# Executive Summary

Traffic congestion inside urban areas is significantly aggravated by drivers roaming parking facilities in search of available parking spaces. This behavior increases fuel consumption, travel time, and local emissions, while also creating unnecessary congestion near commercial centers, universities, and residential complexes.

This project presents the design of an IoT-based Smart Parking System that enables real-time monitoring of parking slot occupancy, remote user access through a mobile application, and automated gate control based on parking availability. The system aims to minimize search time for parking spaces by providing drivers with accurate, up-to-date information about free and occupied slots before and during arrival.

The proposed system deploys infrared (IR) proximity sensors at each parking slot to detect vehicle presence. These sensors are connected to local IoT nodes, which transmit occupancy data to a gateway controller. The gateway aggregates slot data and forwards it to a cloud backend using standard IoT communication protocols. A mobile application interface allows users to visualize parking availability in real time and guides them directly to free spaces. In addition, an electromechanical gate actuator is integrated at the parking entrance to automatically permit or deny access depending on slot availability.

The system is designed to be scalable, cost-effective, and easy to deploy, making it suitable for parking facilities of varying sizes. Emphasis is placed on clear physical deployment, well-defined communication paths, and standardized software protocols to ensure reliability and maintainability.

This report focuses on the system architecture, physical layout, sensor and actuator deployment, communication techniques, backend infrastructure, and man-machine interface, supported primarily through schematic figures and diagrams to clearly demonstrate system operation and engineering decisions.

# Name of IoT Vertical Application

Smart Parking System (Smart Environments Vertical)
This project falls under the Smart Environments vertical and aims to improve urban mobility using IoT technologies. It monitors parking slot occupancy in real time to optimize space usage and reduce traffic congestion caused by unnecessary vehicle searching.

The system uses sensors, gateways, cloud processing, and a mobile application to provide live parking availability and automated access control. It supports smart city goals by enhancing transportation efficiency, sustainability, and user experience.

# Functions Performed by the IOT System

The Smart Parking System performs multiple integrated functions that enable real-time parking monitoring, user guidance, and automated access control. The core system functions are illustrated primarily through schematic figures, with concise explanations provided below.

## 6.1 Real Time Parking Slot Occupancy Detection

Function Description:
Each parking slot is equipped with an infrared (IR) proximity sensor mounted at ground or curb level. The sensor continuously detects the presence or absence of a vehicle in the slot.

This function eliminates manual inspection and enables continuous, automated monitoring of the entire parking area.

## 6.2 Multi Slot Data Collection and Local Processing

🔹 Figure 6.2 — Sensor Nodes Aggregation per Parking Zone

This reduces communication overhead and improves system scalability.

## 6.3 Hierarchical Data Aggregation (Star Topology)

🔹 Figure 6.3 — Star Topology Architecture (Slots → Gateway → Cloud)

Function Description:
The system follows a star topology:

- Sensor nodes communicate only with the gateway

- No node-to-node communication exists

Advantages (Shown in Figure):

- Predictable latency

- Simplified routing

- Low power consumption

- Fault isolation (failure affects only one zone)

The gateway aggregates all slot data and forwards it to the cloud backend.

## 6.4 Cloud Based Parking Availability Management

🔹 Figure 6.4 — Cloud Backend Processing and Storage

Function Description:
The cloud server receives parking data from gateways and performs:

- Real-time slot availability computation

- Historical data storage

- Parking usage analytics

- API services for mobile application access

Processed Outputs:

- Number of free slots

- Zone-level availability

- Peak usage periods

- Slot utilization statistics

## 6.5 Automated Gate Control Based on Availability

🔹 Figure 6.5 — Automatic Gate Control Workflow

Function Description:
An electromechanical gate actuator is installed at the parking entrance.

Control Logic (Shown in Figure):

- If available slots > 0 → Gate opens automatically

- If parking full → Gate remains closed + warning displayed

The gate controller receives commands from the cloud or local gateway to ensure safe and controlled access.

## 6.6 Mobile Application User Guidance

🔹 Figure 6.6 — Mobile Application Interface Workflow

Function Description:
Drivers interact with the system through a mobile application that provides:

- Real-time visualization of free and occupied slots

- Parking zone map

- Slot availability count

- Entry permission status

The application communicates with the cloud backend using secure REST/MQTT services.

## 6.7 Alerting and Notification System

🔹 Figure 6.7 — Event and Alert Generation

Function Description:
The system generates alerts for abnormal conditions such as:

- Sensor failure

- Communication loss

- Parking full condition

- Gate malfunction

Alerts are logged in the backend and optionally pushed to administrators via mobile notifications.

## 6.8 System Scalability and Expansion

🔹 Figure 6.8 — Scalable Smart Parking Deployment

Function Description:
The modular architecture allows:

- Adding new parking slots without system redesign

- Expanding to multiple parking floors or facilities

- Supporting multiple parking locations under one cloud platform

Each new slot or zone integrates seamlessly into the existing system.

## 6.9 Summary of System Functions

🔹 Figure 6.9 — Functional Overview Diagram

The IoT-based Smart Parking System performs:

- Automated slot detection

- Centralized data aggregation

- Cloud-based availability management

- User guidance via mobile application

- Automated gate access control

- Fault detection and alerting

All system functions are implemented using standardized IoT components and communication protocols, ensuring reliability, scalability, and real-world deployability.

# TYPE OF DATA COLLECTED

This section describes all data generated, processed, and transmitted by the IoT-based Smart Parking System.
For clarity and evaluation purposes, each data category is accompanied by its corresponding communication frame format, including headers and error-checking fields.

## 7.1 Parking Slot Sensor Data

Each parking slot is equipped with an IR-based occupancy sensor connected to a local IoT sensor node.
The sensor node periodically reports slot status to the gateway.

Table 1 Parking Slot Sensor Data Specifications

| Parameter | Data Format | Size (Bytes) | Description |
| --- | --- | --- | --- |
| Slot ID | Unsigned Integer | 2 | Unique parking slot identifier |
| Timestamp | Unix Epoch | 4 | Time of Measurement |
| Occupancy Status | Boolean | 1 | 0 = Free, 1 = Occupied |
| Sensor Health | Boolean | 1 | 0 = OK, 1 = Fault |
| Battery Voltage | Float | 4 | Node power status |

Total Payload Size = 12 Bytes

## Parking Slot Event Data

Slot status is also reported event-driven whenever a vehicle enters or exits.

Table 2 Parking Slot Event Data

| Parameter | Size (Bytes) | Description |
| --- | --- | --- |
| Event ID | 1 | Entry / Exit |
| Slot ID | 2 | Affect slot |
| Timestamp | 4 | Event time |
| Duration | 2 | Occupancy duration (seconds) |

Payload Size = 9 Bytes

## Gateway Aggregated Data

Each gateway aggregates data from multiple parking slots within its zone.

Table 3 Gateway Aggregated Data

| Parameter | Size (Bytes) | Description |
| --- | --- | --- |
| Gateway ID | 1 | Zone identifier |
| Timestamp | 4 | Aggregation time |
| Active Slots | 1 | Number of slots reporting |
| Slot Status Bitmap | N bits | Occupancy map |
| Fault Flags | 2 | Slot fault indicators |

## Central Node to Cloud Data

The central node consolidates all gateways’ data and forwards it to the cloud.

Table 4 Central Node to Cloud Data Transmission

| Data Format | Format | Frequency |
| --- | --- | --- |
| Slot Snapshot | JSON | Every 15 min |
| Event Logs | JSON | Event-based |
| System Health | JSON | Hourly |
| Alerts | JSON | Immediate |

## Example JSON Slot Snapshot

{

"parkingId": "PARK-001",

"timestamp": 1701456789,

"slots": [

{"slotId": 12, "status": "free"},

{"slotId": 13, "status": "occupied"}

]

}

## Transmission Protocol Summary

The Smart Parking System uses MQTT as the application-layer protocol for data exchange between the central node and the cloud backend. MQTT is well-suited for IoT deployments that require lightweight messaging, reliable delivery, and scalable access for multiple clients such as dashboards and mobile applications.

Table 5 MQTT Advantages

| Feature | Benefit |
| --- | --- |
| Publish/Subscribe Model | Scalable multi-client access |
| QoS 1 Delivery | Guaranteed Delivery |
| Lightweight Header | Ideal for constrained networks |
| Persistent Sessions | Data buffered during outages |

The MQTT broker is hosted in the cloud environment, while MQTT clients run on the central parking controller, ensuring efficient and secure bidirectional communication.

## Daily Data Volume Estimation

Data generated by the Smart Parking System includes periodic parking slot snapshots, event-driven occupancy changes, alerts, and system health reports.

Table 6 Daily Data Volume

| Data Type | Size per Push | Frequency | Daily Total |
| --- | --- | --- | --- |
| Parking Slot Snapshot | ~4 KB | Every 5 min (288/day) | ~1.15 MB/day |
| Slot Event Uploads (Entry/Exit) | ~0.5 KB | Event-based (~500/day typical) | ~250 KB/day |
| Alerts | Variable | event-based | 10–30 KB/day |
| System Health Reports | ~1 KB | hourly (24/day) | 24 KB/day |

Estimated total uplink traffic: ~1.4–1.5 MB/day

## Cloud Download Data

The cloud backend sends control commands, configuration updates, and firmware upgrades to the parking system.

Table 7 Cloud Download

| Payload | Avg Size | Frequency | Daily Total |
| --- | --- | --- | --- |
| Configuration Updates (Thresholds, Timers) | ~2 KB | Hourly | ~48 KB/day |
| Remote Commands (Maintenance, Reset) | <10 KB | Rare | Negligible |
| Firmware Updates | 20–50 MB | Quarterly | Infrequent burst |

## Average Monthly Data Usage

| Direction | Monthly Average |
| --- | --- |
| Upload | ~40 MB |
| Download | ~3-5 MB |
| Total | ~50 MB/month |

This traffic volume fits comfortably within low-cost commercial IoT data plans.

## Bandwidth & Storage Evaluation

Figure 7.5 illustrates the end-to-end data flow in the Smart Parking System, from parking slot sensors to gateway, central controller, and finally to the cloud platform.

Figure 7.5 -> (Data summarized visually from sensor → gateway → central → cloud)

Table 8 System-Wide Data Flow & Storage

| Source | Daily Volume | Monthly Volume | Annual Volume | Storage Location |
| --- | --- | --- | --- | --- |
| Slot Sensor Data | 0.25 MB | 7.5 MB | 90 MB | Local cache + Cloud DB |
| Slot Event Logs | 0.4 MB | 12 MB | 144 MB | Cloud DB |
| Gateway Aggregated Data | 0.3 MB | 9 MB | 108 MB | Transit only |
| Cloud Upload | 1.5 MB | 45 MB | 540 MB | MQTT Broker → Database |
| Cloud Download | 0.15 MB | 4.5 MB | 54 MB | Temporary |
| Total Network Traffic | ~2.6 MB/day | ~78 MB/month | ~936 MB/year | Distributed |
| Cloud Stored (Long-Term) | ~1.1 MB/day | ~33 MB/month | ~396 MB/year | Database + Object Storage |
| Source | Daily Volume | Monthly Volume | Annual Volume | Storage Location |

## Cloud Storage Cost Projection

The Smart Parking System requires modest cloud storage resources due to the compact nature of sensor and event data.

Table 9 Cloud Storage Cost Projection

| Storage Type | Cost |
| --- | --- |
| AWS S3 Standard | $0.023/GB/month |
| Estimated long-term storage (~400 MB/year) | ≈ $0.11/year |
| RDS instance (500 GB capacity) | $60–100/month |

A single database instance can support multiple parking facilities, reducing per-deployment cost significantly.

## Bandwidth Optimization Techniques

Table 10 Bandwidth Optimization Techniques

| Technique | Effect |
| --- | --- |
| Edge Aggregation | Reduces cloud traffic by combining slot data at gateway |
| Event-Driven Reporting | Eliminates redundant periodic updates for unchanged slots |
| Data Compression | Achieves 60–70% payload size reduction |
| Scheduled Uploads | Shifts non-critical uploads to off-peak hours |

# Information Presentation to End-User

The Smart Parking System provides multi-layer information presentation to ensure that both on-site operators and remote users can monitor parking availability, receive alerts, and interact with the system efficiently.
Information is presented through three complementary interfaces:

- Local Touchscreen Dashboard (on-site HMI)

- Mobile Application (remote user access)

- AI Conversational Assistant (decision support)

This layered presentation guarantees real-time visibility, redundancy, and usability under different operational scenarios.

## 8.1 Central Node Touchscreen Dashboard (On-Site Interface)

The touchscreen dashboard acts as the primary Human–Machine Interface (HMI) deployed at the parking facility control room.
It is directly connected to the Central Node, enabling real-time visualization, manual override, and alert acknowledgement without dependence on internet connectivity.

Table 11 Hardware Configuration of On-Site HMI [DataSheet]

| Component | Specification |
| --- | --- |
| Controller | Raspberry Pi 5 |
| CPU | Cortex-A76 Quad-Core @ 2.4 GHz |
| RAM | 4 GB |
| Display | 10.1″ Capacitive Touchscreen |
| Resolution | 1280 x 800 |
| Protection | IP65 waterproof enclosure |
| Mounting | Adjustable eye level standing |

Table 12 Software Architecture

| Layer | Technology |
| --- | --- |
| OS | Raspberry Pi OS (Debian) |
| Backend | Python 3.11 + Flask REST server |
| Frontend | React.js Web UI |
| Local Storage | SQLite (rolling 30-day buffer) |
| Access Mode | Chromium kiosk full-screen UI |

🔹 Figure 8.1 — On-Site Dashboard Layout

Figure  Mobile Dashboard Overview

The dashboard is visually divided into six functional regions:

┌─────────────────────────────────────────────┐

│ Status Bar (Connectivity, Alerts, System)  │

├─────────────────────────────────────────────┤

│ Parking Overview (Total Slots, Free Slots) │

├─────────────────────────────────────────────┤

│ Slot Status Grid (Color-coded map)          │

├─────────────────────────────────────────────┤

│ Detailed Slot View (Selected Slot)          │

├─────────────────────────────────────────────┤

│ Alert Panel                                 │

├─────────────────────────────────────────────┤

│ Manual Control & Overrides                  │

└─────────────────────────────────────────────┘

Color Coding Used:

- Green Free Slot

- Red Occupied Slot

- Black Offline / Sensor Fault

Selecting a slot opens a detailed panel showing:

- Occupancy state

- Timestamp of last update

- Sensor health

- Manual override (gate control, reset sensor)

## 8.2 Mobile Application Interface (Remote User Access)

The mobile application provides remote access for drivers and parking administrators.
It connects to the cloud backend using secure HTTPS and WebSocket channels, enabling real-time updates without local network access.

### 🔹 Figure 8.2 — Mobile Application Screens

┌───────────────┐

│ Home Screen   │ → Overall parking availability

├───────────────┤

│ Map View      │ → Visual layout of parking slots

├───────────────┤

│ Slot Details  │ → Selected slot information

├───────────────┤

│ Alerts        │ → Notifications & warnings

└───────────────┘

Mobile App Capabilities:

- Real-time display of free/occupied slots

- Navigation guidance to nearest available slot

- Gate access request (if availability confirmed)

- Alert reception (slot changes, congestion)

This interface minimizes unnecessary vehicle roaming, reducing traffic congestion and fuel consumption.

## 8.3 AI Conversational Assistant

The system integrates an AI conversational assistant that allows users to interact with parking data using natural language.

| Query Type | Example |
| --- | --- |
| Status | “Is parking available now ?” |
| Slot-Specific | “Is Slot 24 free ?” |
| Predictive | “Will parking be full in 30 minutes ?” |
| Control | “Open the gate” |
| Analytics | “Peak hours today ?” |

### 🔹 Figure 8.3 — AI Assistant Interaction Flow

User Query

↓

Natural Language Processing

↓

Cloud Analytics / Prediction

↓

Structured Response

↓

Text / Voice Output

The assistant supports English and Arabic, maintains conversational context, and extracts insights from historical occupancy data.

## 8.4 Alert Notification & Escalation System

To guarantee operational safety and availability, the system employs a multi-channel alerting mechanism.

### Table 14 – Alert Notification Channels

| Channel | Trigger Scope |
| --- | --- |
| Dashboard UI Banner | All alerts |
| Push Notification | High & Critical alerts |
| SMS Failover | Unacknowledged critical alerts |
| Email Digest | Daily summary & warnings |

### Table 15 – Automated Intervention Examples

| Condition | Automated Action |
| --- | --- |
| Slot sensor unresponsive | Sensor reset + alert |
| Parking full | Gate access denied |
| Abnormal congestion | Alert + traffic redirection |
| Communication loss | Local cache + retry logic |

### 🔹 Figure 8.4 — Alert Escalation Logic

Event Detected

↓

Severity Classification

↓

Immediate UI Alert

↓

Push Notification

↓

SMS Escalation (if unacknowledged)

This layered escalation ensures no critical event is ignored, even during network outages.

# Detailed Schematic Diagram for the Overall System

This section presents a complete, end-to-end description of the physical deployment, sensing infrastructure, communication links, control logic, backend integration, and user interaction of the Smart Parking IoT System.
All components shown in the schematic diagrams are explicitly mapped to their physical location, hardware/software implementation, and communication protocols, in accordance with course requirements.

## 9.1 Physical Deployment Layout

As shown in Figure 9.1, the Smart Parking deployment covers a parking facility of approximately 60 m × 35 m (total area ≈ 2,100 m²).
The facility contains 50 parking slots, arranged in 5 rows × 10 columns, with sufficient spacing to allow vehicle maneuvering and pedestrian access.

Each parking slot is equipped with an individual IoT sensor node embedded at ground level near the front boundary of the slot.

## 9.2 Zonal Partitioning

To improve scalability and fault isolation, the parking area is logically subdivided into five operational zones, each managing 10 parking slots, as summarized in Table 16.

Table 16 Zone Description

| Zone | Slot Range | Approx. Physical Position |
| --- | --- | --- |
| Zone 1 | Slots 1–10 | Upper-left section |
| Zone 2 | Slots 11–20 | Upper-right section |
| Zone 3 | Slots 21–30 | Middle-left section |
| Zone 4 | Slots 31–40 | Middle-right section |
| Zone 5 | Slots 41–50 | Lower-central section |

## 9.3 Sensor Placement and Radio Coverage

Each parking slot hosts one sensor node (SN) embedded in a weatherproof ground enclosure. The node detects vehicle presence using short-range sensing, ensuring high accuracy while avoiding false positives.

### Wireless Coverage Constraints

- Maximum sensor → gateway distance: ≤ 120 m

- Gateways are positioned to guarantee:

  - Full coverage of all assigned slots

  - Minimal overlap between adjacent zones

  - Redundancy against partial obstruction (vehicles, walls)

Table 17 Zone Coverage

| Zone | Coverage Representation |
| --- | --- |
| Zone 1 | Light Blue |
| Zone 2 | Light Green |
| Zone 3 | Light Yellow |
| Zone 4 | Light Purple |
| Zone 5 | Light Orange |

## 9.4 Central Node & Backhaul Connectivity

The Central Node is installed in a secured control location within the parking facility (control room or security office). It acts as the local system controller and decision-making unit.

- All gateways connect to the Central Node via CAT6 Ethernet cables

- Ethernet links provide:

  - Deterministic latency

  - Zero packet loss

  - Immunity to RF interference

The Central Node establishes backhaul connectivity to the cloud using:

- Primary link: 4G LTE modem

- Fallback link: Fixed broadband (if available)

This guarantees continuous operation even in locations without reliable wired infrastructure.

## 9.5 System Architecture

The Smart Parking system follows a three-tier hierarchical architecture:

- Perception Layer (Sensor Nodes)

  - Detect vehicle presence at each parking slot

- Aggregation Layer (Gateways)

  - Collect and validate data from multiple slots

  - Forward aggregated data upstream

- Application Layer (Central Node + Cloud)

  - Compute availability

  - Manage alerts

  - Provide dashboards and mobile services

This architecture minimizes bandwidth usage, isolates failures, and simplifies system expansion.

## 9.6 Type and Location of Sensors

Each parking slot is equipped with a single-purpose IoT sensor node, resulting in 50 sensor nodes across the facility.

Table 18 Sensor Location

| Sensor Type | Model / Technology | Mounting Location |
| --- | --- | --- |
| Occupancy Sensor | IR Proximity Sensor (TCRT5000 class) | Ground-mounted at slot head |
| Auxiliary Distance Check | Ultrasonic Sensor (HC-SR04) | Slot boundary enclosure |
| Battery Monitor | ADC-based voltage sensing | Inside node enclosure |

## 9.7 Power Budget and Energy Model

Table 19 Baseline Power Characteristics [4] [5]

| Subsystem | Active Current | Avg Duty Time | Avg Consumption |
| --- | --- | --- | --- |
| ESP32 MCU | 80 mA | 20 s / 5 min | ~0.27 mA |
| Wireless TX Burst | 120 mA | 1 s / event | ~0.06 mA |
| IR Sensor | 20 mA | 10 s / 5 min | ~0.07 mA |
| Ultrasonic Sensor | 15 mA | 2 s / 10 min | ~0.005 mA |
| ESP32 MCU | 80 mA | 20 s / 5 min | ~0.27 mA |

Average node current ≈ 0.40–0.45 mA

With a 3000 mAh lithium battery: Lifetime ≃ 7–9 months

With optional solar-assisted charging, operational lifetime exceeds 12 months.

## 9.8 Type & Location of Actuators

Actuators in the Smart Parking system are used to control vehicle access, inform drivers, and enforce availability logic.

Table 20 Actuator Location

| Actuator Type | Function | Physical Placement | Trigger Mechanism |
| --- | --- | --- | --- |
| Entry Gate Barrier | Controls vehicle entry | Parking entrance | Open when free slots available |
| Exit Gate Barrier | Controls vehicle exit | Parking exit | Vehicle detection at exit |
| LED Slot Indicators | Show slot status (Free/Occupied) | Above each slot | Updated by central node |

## 9.9 Type & Location of Gateways — Network Layer Distribution

Five gateway nodes form the aggregation layer of the system.

Table 21 Gateway unction

| Gateway Count | Coverage | Handled Devices | Connectivity |
| --- | --- | --- | --- |
| 5 | 10 slots (sensors) per gateway | 50 sensor nodes | Wireless uplink + Ethernet |

## 9.10 Communication Technique — Sensor Nodes → Gateway

### Sensor Nodes → Gateway

- Protocol: Low-power wireless (LoRa / sub-GHz ISM)

- Topology: Star (NOT mesh)

Advantages:

- Long-range coverage

- Low power consumption

- No routing overhead

- Fault isolation per zone

### Gateway → Central Node

- Protocol: Ethernet (TCP/IP)

- Medium: CAT6 cable

Justification:

- Zero packet loss

- Immunity to RF interference

- Deterministic latency for real-time updates

### Central Node → Cloud

- Protocol: MQTT over TLS

- Internet Link: 4G LTE or fixed broadband

This ensures secure, reliable cloud synchronization even in locations without wired infrastructure

## 9.11 Software Protocol for Data Transfer (Cloud + Edge)

Table 24 MQTT Features [8]

| Feature | Why it is appropriate |
| --- | --- |
| Lightweight | Ideal for periodic parking data |
| Publish/Subscribe | Multiple dashboards and apps |
| QoS Support | Prevents data loss |
| Persistence | Handles temporary outages |

## 9.12 Backend Servers for Storage & Processing

Table 25 Backend Services [9]

| Backend Service | Function |
| --- | --- |
| MQTT Broker | Message routing |
| Cloud Database | Slot status storage |
| Serverless Functions | Event processing |
| Analytics Engine | Usage statistics |
| Notification Service | Driver alerts |

## 9.13 Human Interface Layer (Man–Machine Interaction)

Two primary user interfaces exist:

| Interface | Platform | Role |
| --- | --- | --- |
| Local Dashboard | Central Control PC | Facility monitoring |
| Mobile Application | Android / iOS | Driver guidance |
| Web Portal | Cloud-hosted | Administration & analytics |

# 10. Hardware Components: Brand, Model, and Datasheets

This section documents the hardware components deployed in the Smart Parking IoT System, including sensors, controllers, gateways, and central processing units.
For each component, the brand/model, technical role, and official datasheet reference are provided to ensure traceability, reproducibility, and technical validation.

## 10.1 Parking Slot Sensors

Each parking slot is equipped with a ground-mounted sensing unit responsible for detecting vehicle presence with high reliability under outdoor conditions.

### 10.1.1 Infrared Occupancy Sensor

Table 10.1 — IR Parking Occupancy Sensor

| Parameter | Specification |
| --- | --- |
| Sensor Type | Infrared Reflective Proximity Sensor |
| Model | TCRT5000 |
| Manufacturer | Vishay Semiconductors |
| Detection Principle | IR emitter + phototransistor |
| Operating Voltage | 3.3–5 V |
| Typical Range | 1–15 cm |
| Output Type | Digital / Analog |
| Role in System | Detect vehicle presence at parking slot |

Justification:
The TCRT5000 provides fast response, low power consumption, and high availability, making it suitable for large-scale parking deployments.

### 10.1.2 Ultrasonic Distance Sensor (Redundancy Check)

Table 10.2 — Ultrasonic Distance Sensor

| Parameter | Specification |
| --- | --- |
| Sensor Type | Ultrasonic Ranging Module |
| Model | HC-SR04 |
| Manufacturer | Generic / Multiple vendors |
| Operating Voltage | 5 V |
| Measurement Range | 2 cm – 400 cm |
| Accuracy | ±1 cm |
| Interface | Trigger / Echo GPIO |
| Role in System | Secondary confirmation of vehicle presence |

Justification:
The HC-SR04 is used as a redundant verification sensor to reduce false detections caused by dirt, shadows, or reflective surfaces.

## 10.2 Sensor Node Controller

Each parking slot sensor node is controlled by a low-power microcontroller responsible for sensor sampling, data formatting, and wireless transmission.

### 10.2.1 Microcontroller Unit (MCU)

Table 10.3 — Sensor Node Controller

| Parameter | Specification |
| --- | --- |
| MCU | ESP32-WROOM-32 |
| Manufacturer | Espressif Systems |
| CPU | Dual-core Xtensa LX6 @ 240 MHz |
| RAM | 520 KB SRAM |
| Flash | 4 MB |
| Wireless | Wi-Fi 802.11 b/g/n, BLE |
| Operating Voltage | 3.3 V |
| Role in System | Sensor control, data transmission, power management |

Justification:
The ESP32 provides an excellent balance between processing capability, low power modes, and wireless connectivity, widely adopted in IoT parking systems.

## 10.3 Gateway Hardware

Gateway nodes aggregate data from multiple parking slots and forward it to the central node.

### 10.3.1 Parking Zone Gateway

Table 10.4 — Gateway Node Hardware

| Parameter | Specification |
| --- | --- |
| Gateway Platform | Raspberry Pi Zero W |
| Manufacturer | Raspberry Pi Ltd. |
| CPU | ARM1176JZF-S @ 1 GHz |
| RAM | 512 MB |
| Connectivity | Wi-Fi, USB Ethernet |
| Operating System | Raspberry Pi OS (Linux) |
| Role in System | Slot data aggregation & forwarding |

Justification:
Raspberry Pi Zero W offers Linux-based flexibility, low power consumption, and sufficient performance for gateway aggregation tasks.

## 10.4 Central Node (Edge Controller)

The central node acts as the local control hub, dashboard host, and cloud gateway.

### 10.4.1 Central Processing Unit

Table 10.5 — Central Node Hardware

| Parameter | Specification |
| --- | --- |
| Platform | Raspberry Pi 5 (4 GB) |
| Manufacturer | Raspberry Pi Ltd. |
| CPU | Quad-core ARM Cortex-A76 @ 2.4 GHz |
| RAM | 4 GB LPDDR4X |
| Storage | microSD / NVMe (via PCIe) |
| Connectivity | Gigabit Ethernet, USB 3.0 |
| Role in System | Data aggregation, dashboard, cloud communication |

Justification:
The Raspberry Pi 5 provides sufficient processing headroom for real-time dashboards, database handling, and secure MQTT communication.

## 10.5 Communication Interfaces

Table 10.6 — Communication Modules

| Component | Model | Role |
| --- | --- | --- |
| Wireless Module | ESP32 Integrated Radio | Sensor → Gateway |
| Ethernet Adapter | USB-to-Ethernet | Gateway → Central |
| Cellular Modem (Optional) | Huawei E3372 LTE USB | Central → Cloud |

## 10.6 Summary of Hardware Components

Table 10.7 — Hardware Summary

| Layer | Component | Model |
| --- | --- | --- |
| Sensor Layer | IR Sensor | TCRT5000 |
| Sensor Layer | Ultrasonic Sensor | HC-SR04 |
| Node Controller | MCU | ESP32-WROOM-32 |
| Gateway Layer | SBC | Raspberry Pi Zero W |
| Central Layer | SBC | Raspberry Pi 5 |

# 11. Software Implementation of the Smart Parking IoT System

This section describes the software architecture, modules, and execution logic used to implement the Smart Parking IoT application.
The software stack spans three layers: sensor node firmware, gateway software, and central/cloud-side services. Each layer is designed to be modular, scalable, and fault-tolerant, ensuring reliable operation in real-world parking environments.

## 11.1 Software Architecture Overview

The Smart Parking software follows a hierarchical layered architecture, aligned with the physical system design:

- Sensor Node Firmware (Embedded Layer)

- Gateway Aggregation Software (Edge Layer)

- Central Node & Cloud Services (Application Layer)

Each layer executes a well-defined set of tasks and communicates with adjacent layers using standardized IoT protocols.

## 11.2 Sensor Node Software (Embedded Firmware)

### 11.2.1 Platform and Development Environment

| Item | Description |
| --- | --- |
| MCU | ESP32-WROOM-32 |
| Programming Language | C / C++ |
| Framework | Arduino Core for ESP32 |
| IDE | Arduino IDE / PlatformIO |
| RTOS | FreeRTOS (built-in ESP32 support) |

### 11.2.2 Firmware Responsibilities

Each sensor node firmware is responsible for:

- Reading parking slot sensors (IR + ultrasonic)

- Performing basic validation and filtering

- Packaging data into compact payloads

- Transmitting data wirelessly to the gateway

- Entering low-power sleep modes

### 11.2.3 Firmware Execution Flow

The firmware executes the following loop:

- Initialization Phase

  - GPIO configuration

  - Sensor initialization

  - Network setup (Wi-Fi or LoRa depending on deployment)

  - Load device ID from flash memory

- Data Acquisition

  - Read IR sensor (occupancy detection)

  - Read ultrasonic distance (confirmation)

  - Determine slot state (FREE / OCCUPIED)

- Payload Construction

  - Slot ID

  - Timestamp

  - Occupancy status

  - Sensor health flag

  - Battery voltage

- Data Transmission

  - Send payload to gateway

  - Await acknowledgment (QoS-supported delivery)

- Power Management

  - Enter deep sleep for predefined interval

  - Wake up on timer or event trigger

### 11.2.4 Sensor Node Pseudocode

setup() {

initGPIO();

initSensors();

initNetwork();

}

loop() {

readIRSens();

readUltrasonic();

determineOccupancy();

buildPayload();

sendToGateway();

enterDeepSleep();

}

This approach ensures minimal CPU usage, low power consumption, and predictable timing behavior.

## 11.3 Gateway Software Implementation

### 11.3.1 Platform and Environment

| Item | Description |
| --- | --- |
| Hardware | Raspberry Pi Zero W |
| OS | Raspberry Pi OS (Linux) |
| Programming Language | Python |
| Messaging Protocol | MQTT |
| Broker | Local Mosquitto (optional) |

### 11.3.2 Gateway Responsibilities

The gateway software acts as an aggregation and forwarding layer, performing:

- Reception of sensor node data

- Data validation and timestamp normalization

- Aggregation of multiple parking slots

- Forwarding data to the central node

- Buffering data during connectivity loss

### 11.3.3 Gateway Data Handling Flow

- Subscribe to sensor topics

- Validate payload structure

- Append gateway ID and timestamp

- Store temporarily in memory buffer

- Publish aggregated data to central node

### 11.3.4 Gateway Pseudocode

def on_message(client, userdata, msg):

payload = parse(msg)

validate(payload)

aggregate(payload)

publish_to_central(payload)

This design prevents cloud flooding, reduces bandwidth usage, and isolates failures per zone.

## 11.4 Central Node Software Implementation

### 11.4.1 Platform and Stack

| Component | Technology |
| --- | --- |
| Hardware | Raspberry Pi 5 |
| OS | Raspberry Pi OS |
| Backend | Python (Flask / FastAPI) |
| Database | SQLite (local cache) |
| Dashboard | Web-based UI |
| Cloud Protocol | MQTT over TLS |

### 11.4.2 Central Node Responsibilities

The central node acts as the system brain, executing:

- Aggregation of all gateways’ data

- Parking occupancy computation

- Alert generation (overstay, congestion)

- Local dashboard updates

- Secure cloud synchronization

### 11.4.3 Data Processing Pipeline

- Receive gateway payloads via MQTT

- Store data in local database

- Update real-time dashboard

- Evaluate alert rules

- Forward summarized data to cloud

### 11.4.4 Example Central Processing Logic

if slot_status_changed:

update_database()

update_dashboard()

send_alert_if_needed()

publish_to_cloud()

## 11.5 Cloud-Side Software Services

### 11.5.1 Cloud Platform

| Service | Purpose |
| --- | --- |
| AWS IoT Core | Secure MQTT broker |
| AWS Lambda | Event-driven processing |
| Amazon RDS | Long-term structured storage |
| Amazon S3 | Historical archive |
| Cloud Dashboard | Remote monitoring |

### 11.5.2 Cloud Functions

- Store parking history

- Generate utilization analytics

- Support mobile/web applications

- Enable future AI-based prediction (peak hours, congestion)

## 11.6 Security and Reliability Mechanisms

The software implementation includes:

- TLS encryption for cloud communication

- Device authentication using certificates

- QoS-based message delivery

- Local buffering during outages

- Watchdog timers on embedded nodes

## 11.7 Summary

The Smart Parking software implementation:

- Uses lightweight embedded firmware for scalability

- Employs edge aggregation to reduce bandwidth

- Ensures secure, reliable cloud communication

- Separates concerns across well-defined layers

This modular design simplifies maintenance, future expansion, and real-world deployment.

# 12. Security Services for Safe Operation of the Smart Parking IoT System

This describes the security mechanisms and services required to ensure the confidentiality, integrity, availability, and authenticity of the Smart Parking IoT system.
Given that the system handles real-time occupancy data, control commands, and cloud connectivity, security is essential to prevent unauthorized access, data tampering, service disruption, and privacy violations.

The proposed security model follows a defense-in-depth approach, applying protection at device, communication, gateway, cloud, and user interface levels.

## 12.1 Security Threat Model

The Smart Parking system is exposed to the following potential threats:

- Unauthorized sensor or gateway impersonation

- Eavesdropping on wireless communications

- Data modification during transmission

- Denial-of-Service (DoS) attacks on gateways or cloud services

- Unauthorized access to dashboards or APIs

- Physical tampering with outdoor sensor nodes

Security services are designed to mitigate these risks systematically.

## 12.2 Device-Level Security (Sensor Nodes)

### 12.2.1 Device Authentication

Each sensor node is provisioned with:

- A unique device identifier (Node ID)

- Pre-shared credentials or keys stored in non-volatile memory

Only authenticated devices are allowed to transmit data to the gateway.

### 12.2.2 Firmware Integrity Protection

To prevent malicious firmware modification:

- Firmware is flashed only through secured physical access

- Firmware updates are validated using checksum / hash verification

- Debug interfaces (UART/JTAG) are disabled after deployment

This ensures that only trusted firmware executes on sensor nodes.

### 12.2.3 Physical Security

Sensor nodes are:

- Housed in tamper-resistant enclosures

- Mounted at elevated or enclosed positions

- Designed to reset or invalidate credentials upon unauthorized access attempts (optional enhancement)

## 12.3 Communication Security (Sensor → Gateway)

### 12.3.1 Secure Wireless Communication

Depending on the deployed technology (Wi-Fi or LoRa-based):

- AES-128 encryption is applied at the link or application layer

- Packets include message integrity checks (MIC) to detect tampering

- Replay protection is enforced using timestamps or frame counters

### 12.3.2 Access Control at Gateway

Gateways:

- Accept data only from registered sensor node IDs

- Reject malformed or unauthenticated packets

- Rate-limit incoming messages to mitigate flooding attacks

## 12.4 Gateway and Edge Security

### 12.4.1 Operating System Hardening

Gateways and central nodes run hardened Linux systems with:

- Disabled unused services

- Firewall rules limiting open ports

- Regular OS and security updates

### 12.4.2 Secure Data Forwarding

Data forwarded from gateway to central node uses:

- Authenticated MQTT or TCP sessions

- Controlled topic namespaces

- Role-based access permissions

This prevents unauthorized gateways from injecting data.

## 12.5 Cloud and Backend Security

### 12.5.1 Secure Cloud Connectivity

Communication between the central node and cloud uses:

- MQTT over TLS (Transport Layer Security)

- Server and client authentication using X.509 certificates

- Encrypted data channels (AES-based cipher suites)

This ensures confidentiality and integrity of all transmitted data.

### 12.5.2 Cloud Access Control

Cloud services enforce:

- Role-Based Access Control (RBAC)

- Fine-grained permissions for:

  - Data ingestion

  - Analytics

  - User access

  - Administrative operations

Only authorized services and users can access system resources.

### 12.5.3 Data Protection at Rest

Stored data is protected through:

- Encrypted databases

- Secure cloud storage buckets

- Automated backups with access restrictions

This prevents data exposure in case of unauthorized access.

## 12.6 Application and User Interface Security

### 12.6.1 User Authentication

User access to dashboards and mobile applications requires:

- Secure login credentials

- Token-based authentication (JWT or session tokens)

- Optional multi-factor authentication (MFA)

### 12.6.2 Authorization and Privilege Levels

Users are assigned roles such as:

- Administrator

- Operator

- Viewer

Each role has restricted permissions, preventing accidental or malicious misuse.

## 12.7 Availability and Resilience Measures

To ensure continuous operation:

- Gateways buffer data during connectivity loss

- Central node caches data locally

- Cloud services are fault-tolerant and scalable

- Watchdog timers reset unresponsive nodes

These measures maintain high availability, even during partial failures.

## 12.8 Security Summary

The Smart Parking IoT system integrates security at every layer:

- Device Security: Authentication, firmware protection

- Network Security: Encryption, integrity checks

- Gateway Security: OS hardening, access control

- Cloud Security: TLS, RBAC, encrypted storage

- User Security: Authentication and authorization

This multi-layered security strategy ensures safe, reliable, and trustworthy operation of the Smart Parking IoT system in real-world deployments.

# 13. Prototype Implementation & Hardware Validation

This section describes the physical prototype implementation of the Smart Parking IoT system and explains how the proposed architecture was validated using real hardware components.
The prototype focuses on verifying vehicle detection accuracy, slot occupancy sensing, data aggregation, and system-level decision logic before full-scale deployment.

The implemented prototype corresponds to the layout illustrated in Figure 13.1, which represents a simplified parking area with entry/exit gates, parking slots, distributed sensor nodes, and a central gateway.

## 13.1 Prototype Overview

The prototype emulates a real parking facility using three main functional subsystems:

- Gate Monitoring Subsystem

- Parking Slot Sensing Subsystem

- Gateway Aggregation & Decision Subsystem

Each subsystem is implemented using ESP-based microcontroller nodes and IR sensors, interconnected through a lightweight wireless communication model.

## 13.2 Gate Monitoring Subsystem (Vehicle Counting)

### Hardware Implementation

At the entrance and exit gates, an independent ESP-based controller is deployed, as shown on the left side of the prototype drawing.

Components used:

- ESP microcontroller (gate controller)

- IR obstacle sensor modules (one per gate)

- Power supply (USB / regulated DC)

### Operation Logic

- The entrance IR sensor detects vehicles entering the parking area.

- The exit IR sensor detects vehicles leaving the parking area.

- Each detection event triggers an interrupt or polling-based read on the ESP.

- The ESP updates a vehicle counter:

  - Entry detected → counter increment

  - Exit detected → counter decrement

This mechanism provides a real-time estimation of total vehicles inside the parking area, allowing validation of parking capacity logic.

### Validation Outcome

- Correct increment/decrement behavior was observed during repeated entry/exit tests.

- False triggers were minimized using software debouncing and short detection delays.

## 13.3 Parking Slot Sensing Subsystem

### Slot-Level ESP Distribution

As shown in the drawing, parking slots are grouped such that:

- One ESP microcontroller serves every 5 parking slots

- Each slot is equipped with one IR sensor, mounted facing downward or inward toward the slot

This grouping reduces:

- Hardware cost

- Power consumption

- Network traffic

### Slot Identification

Each parking slot is assigned a unique Slot ID, stored in the firmware of the corresponding ESP.
When an IR sensor detects a vehicle presence:

- Logic 1 → Slot occupied

- Logic 0 → Slot empty

The ESP maintains a local slot status table for its 5 assigned slots.

### Data Packet Structure (Prototype Level)

Each ESP periodically sends a packet containing:

- ESP Node ID

- Slot IDs (1–5)

- Occupancy status bitmask

- Timestamp

This confirms that slot-level identification and reporting are correctly implemented.

## 13.4 Gateway Aggregation Subsystem

### Gateway Role

The gateway (shown in the center of the drawing) performs the following functions:

- Receives data from:

  - Gate ESPs (vehicle count)

  - Slot ESPs (occupancy status)

- Aggregates all incoming information

- Determines:

  - Whether the parking is full or not

  - Which slot IDs are free

- Outputs the system state (for display or cloud forwarding)

### Gateway Hardware

- ESP / Single-board controller (prototype gateway)

- Wireless interface (Wi-Fi or ESP-NOW)

- Optional serial monitor / LCD for debugging

### Gateway Decision Logic

The gateway executes simple validation rules:

- If total occupied slots == total available slots → Parking Full

- Else:

  - Extract and list available Slot IDs

  - Broadcast availability status

This confirms the correct integration between counting logic and slot sensing logic.

## 13.5 Communication Validation

For the prototype phase:

- ESP-to-Gateway communication was implemented using short-range wireless (Wi-Fi / ESP-NOW)

- Packet loss was tested by:

  - Disconnecting individual ESPs

  - Reintroducing them during runtime

The gateway correctly handled:

- Missing updates

- Partial data

- Re-synchronization after reconnection

## 13.6 Physical Validation Results

The prototype successfully validated the following design assumptions:

- ✔ IR sensors reliably detect vehicle presence at slots and gates

- ✔ One ESP can efficiently manage multiple slot sensors

- ✔ Entry/exit counting remains consistent with slot occupancy

- ✔ Gateway aggregation logic correctly reflects parking availability

- ✔ Slot ID-based reporting enables precise user guidance

## 13.7 Prototype Limitations and Scalability

While the prototype is simplified, it demonstrates core feasibility.
In full deployment:

- ESP nodes can be replaced with industrial-grade controllers

- IR sensors can be replaced with ultrasonic or magnetic sensors

- Gateway can forward data securely to cloud services via MQTT

The prototype therefore serves as a functional proof-of-concept validating the system’s architecture and operational logic.

# 14. References

All datasheets and technical references used in this section are listed below in the required academic format.

[1] Vishay Semiconductors, “TCRT5000 Reflective Optical Sensor Datasheet”, Vishay Intertechnology Inc., 2023.

[2] Elecfreaks, “HC-SR04 Ultrasonic Ranging Module Datasheet”, Elecfreaks Manufacturer Documentation, 2022.

[3] Espressif Systems, “ESP32-WROOM-32 Datasheet”, Espressif Systems Co., Ltd., 2023.

[4] Raspberry Pi Ltd., “Raspberry Pi Zero W Product Brief”, Raspberry Pi Trading Ltd., 2021.

[5] Raspberry Pi Ltd., “Raspberry Pi 5 Product Brief”, Raspberry Pi Trading Ltd., 2023.

## Appendix A: Acronyms and Abbreviations

Table 26 summarizes all acronyms used throughout this report for quick reference.

Table 26 Acronyms and Abbreviations[1]

| Acronym | Full Meaning / Definition |
| --- | --- |
| AI | Artificial Intelligence |
| API | Application Programming Interface |
| AWS | Amazon Web Services |
| BNC | Bayonet Neill–Concelman Connector (for pH probe) |
| CPU | Central Processing Unit |
| CSS | Cascading Style Sheets |
| DO | Dissolved Oxygen |
| ESP32-S3 | Espressif Systems Dual-Core MCU Series |
| FCR | Feed Conversion Ratio |
| GB | Gigabyte |
| GPIO | General Purpose Input/Output |
| GSM / LTE | Long-Term Evolution Cellular Network |
| HTML | HyperText Markup Language |
| HTTP/HTTPS | Hypertext Transfer Protocol / Secure TLS Version |
| IoT | Internet of Things |
| IP67 | Ingress Protection Rating (Dust-tight + Immersion Protected) |
| JSON | JavaScript Object Notation |
| LCD | Liquid Crystal Display |
| LF RFID | Low-Frequency Radio-Frequency Identification (125 kHz) |
| LMIC | LoRaWAN MAC Implementation in C |
| LoRa | Long Range Radio Modulation |
| LoRaWAN | Long Range Wide Area Network |
| LSTM | Long Short-Term Memory Neural Model |
| MQTT | Message Queuing Telemetry Transport Protocol |
| NFC (if used later) | Near-Field Communication |
| NTP | Network Time Protocol |
| PCB | Printed Circuit Board |
| pH | Potential of Hydrogen (acidity measure) |
| QoS | Quality of Service Level |
| RAM | Random Access Memory |
| RDS | Relational Database Service (AWS PostgreSQL) |
| RFID | Radio-Frequency Identification |
| RFM95W | LoRa Radio Module (SX1276 chipset) |
| RPM | Revolutions Per Minute (motor/pump units) |
| S3 | Simple Storage Service (AWS) |
| SD Card / microSD | Secure Digital Memory Storage |
| SDK | Software Development Kit |
| SLA | Service Level Agreement |
| SMBus/I2C | Inter-Integrated Circuit Serial Bus |
| SMS | Short Message Service |
| SPI | Serial Peripheral Interface |
| SQL | Structured Query Language |
| TLS | Transport Layer Security |
| UART | Universal Asynchronous Receiver Transmitter |
| UI | User Interface |
| UPS | Uninterruptible Power Supply |
| UUID | Universally Unique Identifier |
| VPN (if added later) | Virtual Private Network |
| Wi-Fi | Wireless Fidelity (local wireless standard) |